Supporting Information

An open normalized drilling exponent for pore-pressure estimation: formulation and blind tests on two public wells

Rajab Ali. All tables below are produced by the scripts in https://github.com/hadree512/nhat-drilling-exponent (commit 122d7bc).

# S1 Formation-pressure tests and predictions (Volve)

Table S1. Every formation-pressure test with the measured gradient and the predictions (sg EMW). n is the number of shale samples in the 50 m (TVD) above the test.

| Wellbore | MD (m) | TVD (m) | Measured | Use | n | d surf. | Dxc serv. | Dxc | N̂ | N̂ P10–P90 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15/9-F-1 | 3332.4 | 3056.0 | 1.137 | offset | 53 | 1.140 | 1.137 | 1.139 | 1.140 | 1.131–1.155 |
| 15/9-F-1 | 3360.8 | 3080.1 | 1.132 | offset | 26 | 1.143 | 1.138 | 1.142 | 1.143 | 1.134–1.159 |
| 15/9-F-1 | 3451.9 | 3162.3 | 1.133 | offset | 15 | 1.136 | 1.138 | 1.136 | 1.136 | 1.127–1.152 |
| 15/9-F-1 | 3462.6 | 3172.2 | 1.136 | offset | 13 | 1.136 | 1.138 | 1.136 | 1.136 | 1.127–1.152 |
| 15/9-F-1 A | 3436.3 | 3147.8 | 1.135 | offset | 11 | 1.139 | 1.139 | 1.139 | 1.138 | 1.129–1.153 |
| 15/9-F-1 A | 3457.5 | 3167.5 | 1.131 | offset | 14 | 1.134 | 1.137 | 1.136 | 1.136 | 1.127–1.152 |
| 15/9-F-1 A | 3462.3 | 3171.9 | 1.131 | offset | 13 | 1.136 | 1.138 | 1.136 | 1.136 | 1.127–1.152 |
| 15/9-F-1 A | 3492.4 | 3200.0 | 1.166 | offset | 9 | 1.136 | 1.138 | 1.137 | 1.136 | 1.127–1.152 |
| 15/9-F-1 B | 3247.0 | 3043.4 | 1.155 | blind | 43 | 1.114 | 1.120 | 1.136 | 1.135 | 1.126–1.151 |
| 15/9-F-1 B | 3255.0 | 3051.2 | 1.153 | blind | 35 | 1.111 | 1.120 | 1.136 | 1.134 | 1.125–1.150 |
| 15/9-F-1 B | 3261.0 | 3057.0 | 1.153 | blind | 29 | 1.111 | 1.120 | 1.136 | 1.136 | 1.127–1.151 |
| 15/9-F-1 B | 3266.5 | 3062.4 | 1.154 | blind | 24 | 1.110 | 1.118 | 1.135 | 1.137 | 1.128–1.153 |
| 15/9-F-1 B | 3273.5 | 3069.3 | 1.153 | blind | 17 | 1.123 | 1.127 | 1.142 | 1.140 | 1.131–1.156 |
| 15/9-F-1 B | 3280.5 | 3076.2 | 1.152 | blind | 10 | 1.111 | 1.119 | 1.137 | 1.142 | 1.134–1.158 |
| 15/9-F-1 B | 3291.0 | 3086.6 | 1.150 | blind | 1 | 1.155 | 1.139 | 1.160 | 1.155 | 1.146–1.170 |
| 15/9-F-1 B | 3296.0 | 3091.5 | 1.150 | blind | 1 | 1.155 | 1.139 | 1.160 | 1.155 | 1.146–1.171 |
| 15/9-F-1 B | 3301.5 | 3096.9 | 1.150 | blind | 1 | 1.156 | 1.139 | 1.160 | 1.155 | 1.146–1.171 |
| 15/9-F-1 B | 3330.0 | 3125.3 | 1.125 | blind | 0 | – | – | – | – | – |
| 15/9-F-1 C | 3273.1 | 2948.3 | 1.170 | blind | 52 | 1.138 | 1.138 | 1.150 | 1.152 | 1.143–1.167 |
| 15/9-F-1 C | 3493.0 | 2980.2 | 1.165 | blind | 112 | 1.127 | 1.132 | 1.143 | 1.144 | 1.135–1.159 |
| 15/9-F-1 C | 3523.2 | 2986.7 | 1.164 | blind | 98 | 1.126 | 1.131 | 1.142 | 1.141 | 1.132–1.157 |
| 15/9-F-1 C | 3551.1 | 2992.9 | 1.164 | blind | 86 | 1.124 | 1.129 | 1.141 | 1.140 | 1.131–1.155 |
| 15/9-F-1 C | 3603.0 | 3006.7 | 1.171 | blind | 43 | 1.128 | 1.132 | 1.144 | 1.148 | 1.139–1.163 |
| 15/9-F-1 C | 3620.2 | 3012.3 | 1.166 | blind | 36 | 1.147 | 1.143 | 1.154 | 1.156 | 1.148–1.172 |
| 15/9-F-1 C | 3720.1 | 3050.6 | 1.164 | blind | 12 | 1.148 | 1.145 | 1.155 | 1.157 | 1.148–1.172 |
| 15/9-F-1 C | 3739.9 | 3057.6 | 1.163 | blind | 8 | 1.149 | 1.146 | 1.155 | 1.157 | 1.148–1.172 |
| 15/9-F-1 C | 3768.1 | 3067.0 | 1.162 | blind | 3 | 1.141 | 1.139 | 1.150 | 1.153 | 1.144–1.169 |
| 15/9-F-1 C | 3823.1 | 3084.9 | 1.160 | blind | 0 | – | – | – | – | – |
| 15/9-F-1 C | 3858.0 | 3096.5 | 1.160 | blind | 4 | 1.133 | 1.135 | 1.145 | 1.143 | 1.134–1.159 |
| 15/9-F-1 C | 3925.1 | 3120.2 | 1.154 | blind | 5 | 1.133 | 1.135 | 1.145 | 1.143 | 1.134–1.159 |
| 15/9-F-1 C | 3977.0 | 3138.8 | 1.152 | blind | 5 | 1.133 | 1.135 | 1.145 | 1.143 | 1.134–1.159 |

# S2 Sensitivity to the normal-trend interval

Table S2. Blind RMSE (sg) for different TVD intervals of the calibration well used to fit the normal trend.

| Interval (m TVD) | d, surface RPM | d, bit RPM | Dxc, service company | Dxc, downhole ECD | N̂ | Offset mean |
| --- | --- | --- | --- | --- | --- | --- |
| 350-700 | 0.161 | 0.078 | 0.027 | 0.134 | 0.075 | 0.021 |
| 350-950 | 0.031 | 0.027 | 0.027 | 0.016 | 0.015 | 0.021 |
| 600-950 | 0.037 | 0.029 | – | 0.014 | 0.013 | 0.021 |
| 350-1350 | 0.031 | 0.028 | 0.144 | 0.016 | 0.015 | 0.021 |
| 350-2300 | 0.032 | 0.049 | 0.132 | 0.130 | 0.135 | 0.021 |

# S3 FORGE 16A(78)-32 bit runs

Table S3. Bit runs parsed from the daily drilling reports, motor ratings and fitted wear coefficients kj (Eq. 8; per 106 revolutions; granite fit). Runs shorter than 5 ft and coring runs are not used.

| Bit | Size (in) | TFA (in2) | In (ft MD) | Out (ft MD) | Motor (rev/gal) | Rating source | kj |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 17.50 | 1.726 | 0 | 1629 | – | – | – |
| 3 | 12.25 | 0.920 | 1629 | 1644 | – | – | – |
| 4 | 12.25 | 1.490 | 1644 | 4552 | 0.155 | stated in report | -4.17 |
| 5 | 12.25 | 1.490 | 4552 | 4964 | 0.155 | stated in report | -8.12 |
| 6 | 12.25 | 1.920 | 4964 | 5113 | 0.155 | inferred from motor size | -0.96 |
| 9 | 8.75 | 0.910 | 5200 | 5345 | 0.242 | inferred from motor size | -6.45 |
| 10 | 8.75 | 0.900 | 5345 | 5473 | 0.242 | inferred from motor size | -5.84 |
| 11 | 8.75 | 1.760 | 5473 | 5495 | – | – | – |
| 14 | 8.75 | 0.910 | 5505 | 5846 | 0.242 | inferred from motor size | -7.56 |
| 18 | 8.75 | 0.840 | 5878 | 6016 | 0.242 | inferred from motor size | 0.00 |
| 22 | 8.75 | 0.830 | 6526 | 6945 | 0.242 | stated in report | -5.07 |
| 23 | 8.75 | 0.840 | 6945 | 7390 | 0.242 | inferred from motor size | -2.91 |
| 29 | 8.75 | 0.840 | 7390 | 8004 | 0.242 | inferred from motor size | 0.00 |
| 31 | 8.75 | 0.830 | 8024 | 8241 | 0.242 | inferred from motor size | -11.62 |
| 32 | 8.75 | 0.840 | 8241 | 8535 | 0.242 | inferred from motor size | 0.00 |
| 34 | 8.75 | 0.840 | 8535 | 9064 | 0.242 | inferred from motor size | 0.00 |
| 35 | 8.75 | 0.840 | 9064 | 9748 | 0.242 | inferred from motor size | -0.90 |
| 37 | 8.75 | 0.840 | 9748 | 10490 | 0.242 | inferred from motor size | -1.40 |
| 38 | 8.75 | 0.840 | 10490 | 10955 | 0.242 | inferred from motor size | -0.63 |
| 45 | 8.75 | 1.374 | 10973 | 10987 | – | – | – |

# S4 Implementation of the method

The full core module is listed below as it stands at commit 122d7bc. Loaders for the two datasets (nhat/io_forge.py, nhat/io_volve.py) and the study scripts are in the repository.

Listing S1. nhat/core.py

"""

Core N-hat (normalized drilling exponent) calculations.

Field units are used throughout:

ROP ft/h, rotary speed rev/min, WOB klbf, bit diameter in,

mud weight / ECD lbm/gal, flow gal/min, pressure psi, TFA in^2, depth ft.

The method in short

-------------------

1.  The classic d-exponent (Jorden and Shirley 1966) and its mud-weight

corrected form Dxc (Rehm and McClendon 1971).

2.  The ROP is mapped to reference drilling conditions before the exponent is

formed:

R* = R * (N_ref / N_bit)^alpha            rotary-speed response

* (min(HSI/HSI_ref, 1))^(-beta)    insufficient bit hydraulics

* exp(-g_ss*SSI - g_wh*WI)          vibration (torque / WOB CV)

* exp(-k_run * n_rev)               bit wear within each run

alpha, beta, g_ss and g_wh are fitted on a calibration interval with a

robust (Huber) regression of ln R; k_run is fitted run by run from the

residual drift against cumulative bit revolutions.

3.  N-hat = log10(R*/(60 N_ref)) / log10(12 W / (1e3 D_b)) * (rho_n / ECD)

4.  The normal trend N-hat_0 is a median (quantile) regression of ln N-hat on

TVD over the calibration interval; the 10th/90th residual quantiles give an

uncertainty band that is carried through Eaton's equation to P10/P90 pore

pressure.

"""

from dataclasses import dataclass, field

import numpy as np

import pandas as pd

from sklearn.linear_model import HuberRegressor, QuantileRegressor

@dataclass

class NHatConfig:

rho_normal_ppg: float = 8.6        # normal pore-fluid gradient as mud weight

rpm_ref: float = 100.0             # reference bit speed

hsi_ref: float = 2.5               # hydraulics above this are taken as sufficient

nozzle_cd: float = 0.95            # nozzle discharge coefficient

eaton_n: float = 1.2               # Eaton exponent for drilling exponents

obg_psi_ft: float = 1.0            # overburden gradient if no density log

alpha_bounds: tuple = (0.3, 1.2)

dyn_window_ft: int = 30            # rolling window for vibration indices

min_run_points: int = 40           # minimum feet per bit run to fit wear drift

band_quantiles: tuple = (0.10, 0.90)

warn_ratio: float = 0.85

severe_ratio: float = 0.70

persistence_ft: int = 30           # an alarm must persist this many feet

# switches for ablation studies (all on = full N-hat)

use_bit_rpm: bool = True           # surface + motor speed instead of surface only

use_alpha: bool = True             # fitted rotary-speed exponent (off = 1, as in d)

use_hydraulics: bool = True

use_dynamics: bool = True

use_wear: bool = True

fitted: dict = field(default_factory=dict)

# ---------------------------------------------------------------- classic

def d_exponent(rop, rpm, wob_klbf, bit_in):

"""Jorden & Shirley (1966): d = log10(R/60N) / log10(12W/10^6 D), W in lbf."""

rop, rpm, wob, bit = (np.asarray(x, float) for x in (rop, rpm, wob_klbf, bit_in))

with np.errstate(divide="ignore", invalid="ignore"):

num = np.log10(rop / (60.0 * rpm))

den = np.log10(12.0 * wob / (1000.0 * bit))

d = num / den

bad = (rop <= 0) | (rpm <= 0) | (wob <= 0) | (den > -0.05) | ~np.isfinite(d)

return np.where(bad, np.nan, d)

def dxc(d, ecd_ppg, rho_normal_ppg):

"""Rehm & McClendon (1971) mud-weight correction."""

return np.asarray(d) * rho_normal_ppg / np.asarray(ecd_ppg, float)

# ---------------------------------------------------------- bit quantities

def bit_rpm(surface_rpm, motor_rpm=None):

"""Bit speed = surface rotation + positive-displacement motor output."""

s = np.nan_to_num(np.asarray(surface_rpm, float))

if motor_rpm is None:

return s

return s + np.nan_to_num(np.asarray(motor_rpm, float))

def bit_hydraulics(mw_ppg, flow_gpm, tfa_in2, bit_in, cd=0.95):

"""Bit pressure drop (psi) and hydraulic horsepower per square inch.

dP_bit = rho Q^2 / (12031 Cd^2 A^2)   (Bourgoyne et al. 1986, Eq. 4.84)

HSI    = dP_bit Q / 1714 / (pi D^2 / 4)

"""

rho, q, a, d = (np.asarray(x, float) for x in (mw_ppg, flow_gpm, tfa_in2, bit_in))

with np.errstate(divide="ignore", invalid="ignore"):

dp = rho * q ** 2 / (12031.0 * cd ** 2 * a ** 2)

hsi = dp * q / 1714.0 / (np.pi * d ** 2 / 4.0)

return dp, hsi

def run_revolutions(depth_ft, rop_fph, bit_rpm_, run_id):

"""Cumulative bit revolutions inside each bit run (time = depth / ROP)."""

df = pd.DataFrame({"z": depth_ft, "r": rop_fph, "n": bit_rpm_, "run": run_id})

dz = df["z"].diff().clip(lower=0).fillna(0)

minutes = 60.0 * dz / df["r"].where(df["r"] > 0)

revs = (df["n"] * minutes).fillna(0)

return revs.groupby(df["run"]).cumsum().to_numpy()

def vibration_indices(torque, wob, window):

"""Rolling coefficient of variation of surface torque (stick-slip proxy)

and of WOB (axial / whirl proxy)."""

t = pd.Series(np.asarray(torque, float))

w = pd.Series(np.asarray(wob, float))

ssi = t.rolling(window, center=True, min_periods=5).std() / t.rolling(

window, center=True, min_periods=5).mean().abs()

wi = w.rolling(window, center=True, min_periods=5).std() / w.rolling(

window, center=True, min_periods=5).mean().abs()

return ssi.clip(0, 1).fillna(0).to_numpy(), wi.clip(0, 1).fillna(0).to_numpy()

# ------------------------------------------------------------- the method

class NHat:

"""Fit-and-apply implementation of the N-hat exponent.

Required columns: MD_ft, TVD_ft, ROP_fph, RPM_surface, WOB_klbf, Bit_in,

MW_ppg.  Optional: Motor_rpm, ECD_ppg, Flow_gpm, TFA_in2, Torque, Run.

"""

def __init__(self, config=None):

self.cfg = config or NHatConfig()

# .................................................. feature building

def prepare(self, df):

c = self.cfg

df = df.copy()

motor = df.get("Motor_rpm") if c.use_bit_rpm else None

df["N_bit"] = bit_rpm(df["RPM_surface"], motor)

ecd = df["ECD_ppg"] if "ECD_ppg" in df else df["MW_ppg"]

df["d"] = d_exponent(df["ROP_fph"], df["N_bit"], df["WOB_klbf"], df["Bit_in"])

df["d_surface_rpm"] = d_exponent(df["ROP_fph"], df["RPM_surface"].clip(lower=1),

df["WOB_klbf"], df["Bit_in"])

df["Dxc"] = dxc(df["d"], ecd, c.rho_normal_ppg)

if {"Flow_gpm", "TFA_in2"}.issubset(df.columns):

df["dP_bit_psi"], df["HSI"] = bit_hydraulics(

df["MW_ppg"], df["Flow_gpm"], df["TFA_in2"], df["Bit_in"], c.nozzle_cd)

else:

df["dP_bit_psi"], df["HSI"] = np.nan, c.hsi_ref

df["hyd_term"] = np.log(np.clip(df["HSI"].fillna(c.hsi_ref) / c.hsi_ref, 0.05, 1.0))

if "Torque" in df:

df["SSI"], df["WI"] = vibration_indices(df["Torque"], df["WOB_klbf"], c.dyn_window_ft)

else:

df["SSI"], df["WI"] = 0.0, vibration_indices(df["WOB_klbf"], df["WOB_klbf"],

c.dyn_window_ft)[1]

run = df["Run"] if "Run" in df else pd.Series(0, index=df.index)

df["n_rev"] = run_revolutions(df["MD_ft"], df["ROP_fph"], df["N_bit"], run)

df["Run"] = run

return df

# ............................................................ fitting

def fit(self, df, calib_mask):

"""Fit the response coefficients on the calibration interval."""

c = self.cfg

ok = calib_mask & np.isfinite(df["d"]) & (df["N_bit"] > 0)

X = np.column_stack([

df["TVD_ft"] / 1e4,

np.log(12 * df["WOB_klbf"] / (1000 * df["Bit_in"])),

np.log(df["N_bit"].clip(lower=1) / c.rpm_ref),

df["hyd_term"],

df["SSI"],

df["WI"],

])[ok]

y = np.log(df["ROP_fph"])[ok]

names = ["depth", "ln_w", "alpha", "beta", "g_ss", "g_wh"]

active = list(range(len(names)))

for _ in range(len(names)):

hub = HuberRegressor(max_iter=2000).fit(X[:, active], y)

coef = dict(zip([names[i] for i in active], hub.coef_))

# physical sign checks: drop terms that come out with the wrong sign

drop = [n for n in ("beta",) if n in coef and coef[n] < 0]

drop += [n for n in ("g_ss", "g_wh") if n in coef and coef[n] > 0]

if not drop:

break

active = [i for i in active if names[i] not in drop]

fitted = {n: float(coef.get(n, 0.0)) for n in names}

fitted["alpha"] = float(np.clip(fitted["alpha"], *c.alpha_bounds))

fitted["n_calib"] = int(ok.sum())

fitted["r2_calib"] = float(hub.score(X[:, active], y))

# run-wise wear drift from residuals against cumulative revolutions

pred = hub.predict(np.column_stack([df[["TVD_ft"]].to_numpy()[:, 0] / 1e4,

np.log(12 * df["WOB_klbf"] / (1000 * df["Bit_in"])),

np.log(df["N_bit"].clip(lower=1) / c.rpm_ref),

df["hyd_term"], df["SSI"], df["WI"]])[:, active])

resid = np.log(df["ROP_fph"].clip(lower=0.1)) - pred

k = {}

for rid, g in df.assign(res=resid).groupby("Run"):

g = g[np.isfinite(g["res"]) & np.isfinite(g["d"])]

if len(g) >= c.min_run_points and g["n_rev"].max() > 0:

slope = np.polyfit(g["n_rev"] / 1e6, g["res"], 1)[0]

k[rid] = float(min(slope, 0.0))          # only slowing counts as wear

else:

k[rid] = 0.0

fitted["k_run_per_Mrev"] = k

c.fitted = fitted

return fitted

# ........................................................... applying

def transform(self, df):

c, f = self.cfg, self.cfg.fitted

df = df.copy()

k = df["Run"].map(f["k_run_per_Mrev"]).fillna(0.0) if c.use_wear else 0.0

alpha = f["alpha"] if c.use_alpha else 1.0

beta = f["beta"] if c.use_hydraulics else 0.0

g_ss, g_wh = (f["g_ss"], f["g_wh"]) if c.use_dynamics else (0.0, 0.0)

ln_rstar = (np.log(df["ROP_fph"].clip(lower=0.1))

- alpha * np.log(df["N_bit"].clip(lower=1) / c.rpm_ref)

- beta * df["hyd_term"]

- g_ss * df["SSI"] - g_wh * df["WI"]

- k * df["n_rev"] / 1e6)

df["ROP_star"] = np.exp(ln_rstar)

ecd = df["ECD_ppg"] if "ECD_ppg" in df else df["MW_ppg"]

df["N_hat"] = d_exponent(df["ROP_star"], c.rpm_ref, df["WOB_klbf"],

df["Bit_in"]) * c.rho_normal_ppg / ecd

# individual correction factors as multipliers on Dxc (for reporting)

df["f_total"] = df["N_hat"] / df["Dxc"]

return df

# ................................................ trend and pressure

def baseline(self, df, calib_mask, col="N_hat", x="TVD_ft"):

"""Median regression of ln(col) on depth plus a residual-quantile band."""

c = self.cfg

ok = calib_mask & np.isfinite(df[col]) & (df[col] > 0)

X = df.loc[ok, [x]].to_numpy() / 1e4

y = np.log(df.loc[ok, col].to_numpy())

qr = QuantileRegressor(quantile=0.5, alpha=0.0, solver="highs").fit(X, y)

base = np.exp(qr.predict(df[[x]].to_numpy() / 1e4))

res = y - qr.predict(X)

lo, hi = np.quantile(res, c.band_quantiles)

out = {f"{col}_0": base, f"{col}_0_lo": base * np.exp(lo), f"{col}_0_hi": base * np.exp(hi)}

return out, {"intercept": float(qr.intercept_), "slope_per_10kft": float(qr.coef_[0]),

"res_lo": float(lo), "res_hi": float(hi)}

def eaton(self, ratio, tvd_ft, obg_psi_ft=None):

"""Eaton (1975): Pp = S - (S - Pn) (x/x_n)^n, returned as ppg EMW."""

c = self.cfg

obg = c.obg_psi_ft if obg_psi_ft is None else obg_psi_ft

s = obg / 0.052

return s - (s - c.rho_normal_ppg) * np.clip(ratio, 0.2, 2.0) ** c.eaton_n

def alarms(self, ratio):

"""Warning flag that must persist `persistence_ft` consecutive samples."""

c = self.cfg

raw = pd.Series(np.asarray(ratio) < c.warn_ratio)

grp = (raw != raw.shift()).cumsum()

length = raw.groupby(grp).transform("size")

return (raw & (length >= c.persistence_ft)).to_numpy()

# S5 Reproducing the results

Listing S2. Commands that regenerate every table and figure.

python -m venv venv

venv/Scripts/pip install -r requirements.txt

# FORGE: download the mud log, daily reports and survey (URLs in nhat/io_forge.py),

# convert the daily-report PDFs with: pdftotext -layout <pdf> <txt>

python scripts/forge_case.py --data <forge_dir> --out results/forge

# Volve: wells/<wellbore>/{mud_log,mwd_lwd,formation_pressure}/ from the Equinor data village

python scripts/volve_case.py --root <volve_dir> --out results/volve

python scripts/volve_bootstrap.py

python scripts/make_figures.py

Run times are a few minutes on a laptop. The scripts use fixed random seeds, so the bootstrap intervals are reproducible.